Di Wang, Member, IEEE, Yuhui Wang, and Xiaochen Xian, Member, IEEE

Degradation Modeling and Prognostics Via A Data Fusion-Based Multitask Learning Approach

Abstract—Remaining useful lifetime (RUL) prediction based on degradation signals of sensors is of critical importance in prognostics and health management (PHM) of machines in manufacturing systems. The machines are often under the same operational and environmental condition and thus show dependent but heterogeneous degradation profiles. However, many existing studies ignored these characteristics among machines, leading to poor RUL prediction results especially when missing data exists. To address this issue, this paper develops a data fusion-based multi-task learning (MTL) method for degradation modeling and prognostics of machines based on multiple sensor signals. Specifically, we first adopt a quadratic polynomial model to fit the degradation processes of machines based on health index (HI) constructed by a linear fusion model. Considering the dependency and heterogeneity among machines, we innovatively integrate MTL into the degradation model. And for parameter estimation, we develop a unique iterated quadratic optimization-integrated expectation-maximum (QOIEM) algorithm that considers the properties of HI construction and simultaneously updates parameters of the fusion model and the degradation model through quadratic programming (QP) and maximum a posteriori-based expectation-maximum (MAP-EM). The QOIEM has been proved to have satisfactory convergence performance in theory in this study. Numerical experiments and a real case study on aircraft turbine engines have been implemented to compare the prognosis performance and robustness on incomplete dataset for the proposed and benchmark methods.

Note to Practitioners— This paper aims to develop a data fusion-based MTL approach for degradation modeling and prognostics based on multiple sensor signals. The proposed method addresses the following challenges in practice: 1) how to collectively utilize the common information with a group of machines under similar or dependent operational conditions; 2) how to handle missing values in observed signals to achieve satisfactory RUL prediction results.  There are four steps for implementing this method in practice: 1) collecting multiple sensor signals of machines (e.g., aircraft engines); 2) establishing the data fusion-based MTL model that shares common information among machines for degradation modeling and prognostics; 3) conducting the QOIEM algorithm to estimate model parameters; 4) making prognostic predictions for in-service machines. The novelty of the proposed method is that it utilizes the data fusion-based MTL model to characterize the dependency and heterogeneity among machines for better degradation modeling and prognostics. The proposed method has superior prognosis results especially when missing data exists comparing with state-of-the-art benchmarks.

Index Terms—Data fusion-based multitask learning, quadratic optimization-integrated expectation-maximum algorithm, missing data

INTRODUCTION

D

EGRADATION of machines is inevitable in most of manufacturing systems, such as aircraft engines [1], [2], bearing systems [3], [4] and lithium batteries [5], [6]. Prognostics and health management (PHM) of machines by considering their degradation processes is critically important to ensure the stability of the overall manufacturing systems.  The remaining useful lifetime (RUL) prediction of machines is one of the most essential tasks in PHM. Based on accurately predicted RUL, maintenance or replacement of machines can be arranged reasonably.

In existing studies, many physics-based methods have been operated to obtain the prediction of RUL through analyzing the physical processes of machines in manufacturing systems [7], [8]. However, these methods are based on an in-depth understanding of the degradation mechanisms and thus are applicable only to simple systems with known degradation mechanisms. They may encounter trouble in RUL prediction of complex systems with unfamiliar degradation mechanisms. To alleviate this problem, data-driven methods predict the RUL based on collected sensor signals that reflect the degradation status of machines. Single sensor signal-based analysis selects a single type of sensor signals that can mainly reflect the degradation status of corresponding machine and then fit a degradation model based on the selected sensor signals, such as stochastic process models [9], [10] and general path models [11], [12]. However, as single sensor signal only contains partial information about the degradation status, analysis based on single sensor signal fails to characterize the degradation processes of machines in complex systems which have multiple sensors that contain comprehensive information of the

Fig. 1.  Degradation signals of a group of engines in an aircraft.

degradation status, and therefore gains inaccurate estimation of RUL [1], [13]. To fully utilize such information, researchers have focused on analyzing multiple sensor signals in systems.

For multiple sensor signal-based analysis, data fusion is an effective way because it can fuse comprehensive degradation information from multiple aspects of machines for better RUL prediction. Existing data fusion methods can be divided into two categories, i.e., decision-level fusion and data-level fusion. Decision-level fusion focuses on fusion methods of different prediction results. The different prediction results can be provided by considering multiple algorithms [14], multiple failure modes [13] and multiple environmental conditions [15]. The limitation of decision-level fusion is that it regards each sensor signal as a single degradation process and analyzing different sensors separately, owing to which the results are sensitive to the sensor signals that cannot well characterize the degradation status. Data-level fusion focuses on combining information from multiple sensor signals to obtain a more informative fused signal. Such data-level fusion methods in the literature include functional principal components analysis [16], hidden Markov models [17], [18], neuro-fuzzy systems [19]–[21] and neural networks [22], [23].

In particular, health index (HI)-based methods in data-level fusion have received widespread attention in recent years. HI is designed to characterize the underlying degradation processes of machines, and it can also be interpreted as a visualization of machines’ health statuses. Liu et al. [24] designed HI as a weighted sum of multiple sensor signals. Specifically, they proposed several theoretical assumptions to ensure the monotonicity, stability etc. of the constructed HI and then applied these assumptions as constraints in a quadratic programming model to obtain optimal fusion weight coefficient for each sensor. Therefore, the constructed HI has better monotonicity and fuses complete degradation information compared to any single sensor signal, and thus yields a better degradation model by fitting the constructed HI. As further researches, Song et al. [25] and Li et al. [26] proposed new fusion methods to construct HI that has nonlinear relations with sensor signals in complicated systems. These studies enriched the theories and construction methods of HI, and obtained fine prognosis results.

However, in real production processes, a group of machines in a manufacturing system are often under similar or dependent operational and environmental conditions (e.g., from the same engines in the same aircrafts), leading to dependent but possibly heterogeneous degradation profiles of each individual machine.  For example, Fig. 1 illustrate degradation signals of a group of engines in an aircraft,  where the data are originated from the Commercial Modular Aero-Propulsion System Simulation (C-MAPSS) [27]. Since these engines are under the same operational and environmental condition in the aircraft, their degradation processes present similar but not identical profiles, which indicates the dependency and heterogeneity of the engines. Such characteristics may contain key information for HI construction, degradation modeling and RUL prediction, but have been ignored in the above studies. Another challenge lies in that sensor signals for prognostic analysis may not be completely and appropriately gathered in practice for unexpected reasons, e.g., sensor aging, data reading errors, and wireless communication failures, leading to the existence of missing data. Currently, there is a lack of an effective method for degradation modeling and prognostics by considering dependency and heterogeneity among machines especially when missing data exist in sensor signals, though it remains an essential task to be solved.

There have been attempts in the literature that partially addressed the issues with missing data and considered similarities among different data sources. Models such as state-space model [28], naive bayes model [29] and kernel-based data-level fusion model [25] have been applied to time series data with missing values. Meanwhile, interpolation and extrapolation methods have been adopted to fill up incomplete datasets, such as linear interpolation, Kalman filtering [30], neural network [31], autoregressive (AR) predictor [32] and spline interpolation [33]. These methods can be regarded as single-task learning (STL), by which information is learned from only a single data source and thus fail to reflect domain-specific knowledge in related tasks, leading to poor model performance when a large number of data are missing. Instead, multi-task learning (MTL) is designed to share common information between related individual tasks [34]. Yu et al. [35] adopted a gaussian process from multiple tasks to characterize common features among the tasks, and then applied the maximum a posteriori based expectation-maximum (MAP-EM)  algorithm to estimate model parameters. Such capacity in capturing dependency and heterogeneity from related tasks allows MTL to achieve a better performance than STL for handling incomplete datasets. For instance, Wan et al. [36] proposed a Bayesian MTL model to reconstruct the incomplete datasets for structural health monitoring, and validated that the model had better performance than traditional Bayesian STL model. Moreover, MTL has been applied to areas that are related to degradation process, e.g., Alzheimer’s disease (AD) neuroimaging [37], solar panel PV [38], aircraft engines [39] etc. However, these methods either ignore data fusion from multiple sensor signals to construct HI or divide degradation modeling and data fusion into two independent parts, leading to poor RUL prediction results.

To fill the research gap, this paper develops a data fusion-based multi-task learning method for degradation modeling and prognostics of machines based on multiple sensor signals. Specifically, we adopt a quadratic polynomial model to characterize the degradation processes of machines based on the HI constructed by a linear fusion model. Considering the dependency and heterogeneity among machines under the same operational and environmental condition, we innovatively integrate the MTL into the degradation model and assume the model parameters of all machines follow a common multivariate Gaussian distribution. For parameter estimation, we develop a unique quadratic optimization-integrated expectation-maximum (QOIEM) algorithm. In the QOIEM, quadratic programming, which considers the properties for HI construction including minimum fitting errors, minimum variance at the failure time and monotonicity, is integrated into the MAP-EM algorithm to update the parameters in the fusion model and degradation simultaneously. Finally, given the constructed HI and updated model parameters, the RUL of machines can be predicted correspondingly.

The proposed method addresses the challenges of degradation modeling and prognostics in the following three aspects: First, by fully considering the dependency and heterogeneity among machines under the same operational and environmental condition, this paper develops a data-level fusion based multi-task learning method for degradation modeling, which proposes a quadratic polynomial model to characterize the degradation status of machines and assume the model parameters of all machines follow a common multivariate Gaussian distribution. Second, a unique QOIEM algorithm is developed for parameter estimation, which can achieve accurate parameter estimation in the fusion model and the degradation model simultaneously and have satisfactory convergence performance in theory. Third, the proposed method is capable to make prognostic predictions of machines effectively when missing data exist in sensor signals, which has been validated through numerical experiments and real case study on C-MAPSS data [27]. Moreover, the proposed method has superior robustness for accurate RUL prediction under different ratios of missing data compared with state-of-the-art benchmarks.

The remainder of this paper is organized as follows. Section II introduces the proposed data fusion-based multitask learning method. Section III and IV present numerical experiments and a case study of aircraft gas turbine engines to evaluate the performance of the proposed method and compare it with existing benchmarks. Section V provides the conclusion and future work of this paper.

Methodology

Model formulation

We regard a group of units as our research objects, which are able to represent objects (e.g., machines) in degradation processes in real production cases. Following common settings in existing works [9], [11], we define the failure time τl of unit l as the time when the underlying degradation status glt exceeds a predefined failure threshold D:

τl=argmintglt≥D,                        (1)

where the degradation status glt is a function of time t. We consider units operate under similar or dependent operational and environmental condition.

Following [24], we present the definition of HI as follows. We denote xlt=xl1t,⋯,xlmt∈R1×m as a vector of sensor signals collected from m sensors of unit l at time t. We assume the degradation status glt can be recovered from a fusion function zxlt of multiple sensor signals with the contamination of white noises, i.e.,

glt=zxlt-εlt.                            (2)

The fusion function zxlt is a function of sensor signals xlt. εlt denotes the noise of unit l at time t, where εlt~N0,σ2. In particular, we define HI as a combination of multiple sensor signals by the fusion function:

hlt=zxlt,                                (3)

where hlt denotes the HI of unit l at time t. To summarize, the relation between the HI and the underlying degradation status can be expressed as follows:

hlt=zxlt=glt+εlt.                      (4)

Data fusion-based multitask learning for HI construction and degradation modeling

We consider a group of units under dependent operational and environmental conditions and therefore their degradation statuses have dependent profiles. Without loss of generality, we consider L units with similar degradation statuses. For each unit l (l=1, ⋯, L) that has nl observed time points, we adopt the quadratic polynomial degradation model to describe its degradation status based on empirical studies and trend analysis of sensor signals [24]:

glt=ϕtΓl,                                  (5)

where ϕt=1, t, t2∈R1×3 is the vector of quadratic basis function at time t, with t=t1,t2,⋯,tnl, and Γl=γl0,γl1,γl2T ∈R3×1 is the vector of model parameters.

Since HI represents the health status of each unit, we design HI by setting the weight coefficient w∈Rm×1 to linearly superimpose xlt following [24], and thus hlt can be represented as

hlt=zxlt=xltw,                          (6)

where hlt is the HI at time t. The extension to nonlinear fusion functions can be implemented by setting kernel functions or developing new models, which will be the focus of future study and omitted here. According to (4), (5) and (6), the relationship among the HI, multiple sensor signals and degradation status of unit l can be expressed by a matrix form when there are nl observed time points as follows:

hl=xlw=ΦlΓl+ϵl,                             (7)

where  hl=hlt1,hlt2,⋯,hltnlT∈Rnl×1 denotes the vector of hlt, xl=xlt1,xlt2,⋯,xltnlT∈Rnl×m denotes the matrix of xlt, Φl=ϕt1,ϕt2,⋯,ϕtnlT∈Rnl×3 denotes the matrix of ϕt, and ϵl=εlt1,εlt2,⋯,εltnlT∈Rnl×1 denotes the vector of εlt.

To characterize the dependency and heterogeneity among units, we integrate multitask learning into the above model structure, which aims at sharing knowledge gained in individual units. Specifically, we consider that the model parameter Γl of each unit shares common knowledge (i.e., dependency and heterogeneity) among units and assume it follows a multivariate normal distribution:

Γl∼NμΓ,CΓ,                                 (8)

where μΓ and CΓ are mean vector and covariance matrix of the multivariate normal distribution and represent the dependency and heterogeneity between units respectively. According to [35], we assume the hyperprior distribution of μΓ and CΓ is a normal-inverse Wishart (IW) distribution, which is the conjugate prior for the multivariate Gaussian distribution

(μΓ,CΓ)∼NμΓμΓ0,1βCΓIWCΓCΓ0,τ.           (9)

Here, μΓ subjects to multivariate normal distribution NμΓμΓ0,1βCΓ in which μΓ0, 1βCΓ are corresponding mean vector and covariance matrix respectively and β means the level of confidence in the prior distribution. CΓ subjects to IW distribution IWCΓτ,CΓ0 in which CΓ0∈R3×3 is the inverse scale matrix and  τ is the degree of freedom with τ>2.

The data-fusion based MTL model can be concluded as follows:

- μΓ and CΓ are generated based on (9);

- For model parameter Γl of each unit l, we have Γl from its distribution as  Γl∼NμΓ,CΓ, with l=1, ⋯, L;

- Given xl that denotes the matrix of observed sensor signals of unit l, the HI can be constructed as  hl=xlw and then the quadratic model is fitted by hl=ΦlΓl+ϵl.

Here, the estimation of the model parameters will be based on all historical units to achieve the multitask learning for HI construction and degradation modeling.

QOIEM algorithm for parameter estimation

In this section, we propose a QOIEM algorithm to estimate parameters in the data-level fusion based multi-task learning model. There exist some challenges in parameter estimation, i.e., 1) in conventional models, weight coefficient w and the degradation model parameters Θ=μΓ,CΓ,σ2 are interdependent. In other words, only one of them can be estimated while the others are obtained through additional engineering knowledge. In our proposed model, however, they are both unknown parameters; 2) Θ=μΓ,CΓ,σ2 contains multiple parameters, which makes it difficult to estimate parameters via derivation of the joint probability density function (PDF).

To address above challenges and estimate the required parameters, i.e., 1) weight coefficient w in the linear fusion function, 2) mean vector μΓ and covariance matrix CΓ of the multivariate normal distribution NμΓ,CΓ for Γl in the quadratic degradation model, and 3) the variance σ2 of the normal distribution N0,σ2 for the noise term ϵl, we develop the QOIEM algorithm, and its pseudo codes are provided in Algorithm 1.

| Algorithm 1 QOIEM algorithm |  |
| --- | --- |
|  | Input: sensor signals of historical units containing observations until failure |
|  | Output: estimated parameters Θ=μΓ,CΓ,σ2 and w |
| 1: | Procedure: |
| 2: | Initialize parameters Θj=[μΓ(j),CΓ(j),σ2(j)]; j =0 |
| 3: | While |
| 4: | Estimate weight coefficient w(j) in the fusion model through quadratic optimization in (12) |
| 5 | Estimate parameters Θ=μΓ,CΓ,σ2 in the degradation model through MAP-EM algorithm: |
| 6: | E step: obtain expected log-likelihood QΘ,Θj through (13) |
| 7 | M step: update Θ and obtain Θj+1 through arg maxΘ QΘ,Θj following (14) |
| 8: | If convergence is achieved, i.e., RMSEj+1%<δ |
| 9: | Stop the loop |
| 10: | Else |
| 11: | j=j+1 |

According to Algorithm 1, we utilize all of the L historical units with their completely observed sensor signals xl (l=1, …, L) that contain their entire degradation processes until failure as the input of the algorithm, and the procedure of the QOIEM algorithm can be listed as follows, i.e., 1) initialize parameters Θj=[μΓ(j),CΓ(j),σ2(j)] at the beginning of the iterations, i.e., j=0, where j denotes the number of iteration, as μΓ(0)=0, CΓ(0)=I and σ2(0)=0, where I is the identity matrix, 2) estimate the weight coefficient w via quadratic optimization by considering the properties for HI construction, 3) expectation (E) step: first obtain the log-likelihood of joint distribution Phl(j),ΓlΘ based on complete data, and then calculate the expected log-posterior QΘ,Θj over the posterior distribution function of latent variable Γl, 4) maximum (M) step: update Θ to Θj+1 through arg maxΘ QΘ,Θj, and 5) check if the Θj+1 and Θj satisfy the convergence condition. Repeat 2) - 5) until the convergence condition is satisfied.

Quadratic programming to estimate w

Given Θj at iteration j, we estimate the weight coefficient w in the linear fusion function by considering the following principles for HI construction:

1) Minimum fitting errors: The fitting errors between the degradation model and the constructed HI should be minimum [1], as we taking HI as the health status of a unit and it should be well fitted by the degradation model.

2) Minimum variance at the failure time: The variance of the constructed HIs for all historical units at their failure time should be minimum [24]. These units are under similar operational condition and failure mode, and thus we assume they have the same failure threshold.

3) Monotonicity: The constructed HI should follow a monotonic trend at all time points [24], as we suppose the health status of a unit should follow a monotonic trend and it comes to failure when its HI reaches the threshold.

According to these properties for HI construction, we can create the corresponding objective function and constraints. Considering the degradation statuses at time points that are closer to failure time have higher effect on RUL prediction, we set the weight matrix cl for different time points (see details in Appendix A), and write the weighted residual term from (7) as cl⋅ϵl=cl⋅xlw-cl⋅ΦlΓl, where Γl is the posterior expectation of the parameter Γl in the degradation model and can be represented as a function with respect to w.

To obtain the posterior expectation and the covariance of Γl, first we calculate the log-likelihood of joint distribution Phl,ΓlΘ of complete data containing hl and Γl given Θ based on all historical units, and it can be written as

Phl,ΓlΘ=l1Zlexp-12JΓl,                (10)

where

JΓl=1σ2xlw-ΦlΓlTxlw-ΦlΓl+Γl-μΓTCΓ-1Γl-μΓ

and Zl denotes the normalization coefficient for the distribution. Then, the posterior expectation of  Γl  at iteration j can be estimated by setting ∂Jhl,ΓlΘj∂Γl=0 and the covariance of Γl can be estimated by the inverse of the Hessian matrix  ∂JΓl∂Γl∂ΓlT , which leads to

Γl=1σ2ΦlTΦl+CΓj-1-11σ2ΦlTxlw+CΓj-1μΓj,

CΓl=1σ2ΦlTΦl+CΓj-1-1,                      (11)

where Θj=μΓj,CΓj,σ2j is the result of the previous iteration, Γl and CΓl are the estimated posterior expectation and covariance. Therefore, the unbiased estimation of squared fitting errors can be calculated as lclϵlTclϵl/(L-1).

Meanwhile, the unbiased estimation of the variance of the constructed HI at the failure time can be calculated as xτwTGxτw. Here, xτ=xτ1, …,xτl, …,xτLT∈RL×m and xτl denotes the vector of sensor signals of unit l at its failure time τl. Recall that xl=xlt1,xlt2,⋯,xltnlT∈Rnl×m represent sensor signals matrix that contains complete degradation process until failure of a historical unit l, and thus its failure time τl is equal to its last observed time point tnl. G=(I-1T1L)/(L-1)∈RL×L, where I ∈RL×L is the identity matrix and 1∈RL×1 is a unit vector of all ones (see details in Appendix B). Therefore, given the above expressions, we obtain the objective function for HI construction to minimize the fitting errors and the variance at the failure time as follows:

min  1-rlclϵlTclϵlL-1+rxτwTGxτw

s.t.  Nw=1,Mw≥0,

where r is the tuning coefficient that can be determined by k-fold cross-validation. The first constraint uses Nw=1 to normalize the weights of sensors, and N∈R1×m is only composed by 1 and -1 to reflect the increasing or decreasing trend of the corresponding sensor signal. The second constraint Mw≥0 ensures the monotonicity of HI with M=diagN ∈Rm×m and 0∈Rm×1 that denotes the vector of all zeros. Then, we obtain the quadratic programming model for the estimation of the weight coefficient w at iteration j

min 12wTCw+wTD

s.t.  Nw=1,-Mw≤0,                      (12)

where

C=21-rL-1lETcl2E+2rxτTGxτ,

D=2r-1L-1lETcl2F,

with

E=xl-1σ2Φl1σ2ΦlTΦl+CΓj-1-1ΦlTxl,

F=Φl1σ2ΦlTΦl+CΓj-1-1CΓ-1μΓj.

This standard form of quadratic programming (QP) problem can be solved by existing solvers, e.g., the qpOASES package in Python that provides a parametric active-set algorithm for QP [40], and thus the weight coefficient w at iteration j can be estimated as w(j).

EM algorithm to estimate Θ

After the weight coefficient wj is obtained, we adopt the EM algorithm to update the parameters Θ=μΓ,CΓ,σ2 in the degradation model. We take hlj=xlw(j) as the observed data and Γl as the latent variable. In EM algorithm, we suppose to calculate the expected log-likelihood of complete data in the E step and update the parameters in the M step.

The E step: we are supposed to calculate the expected log-posterior QΘ,Θjbased on the joint distribution Phlj,ΓlΘ  and hyperprior distribution (i.e., (9)) function P(Θ) according to the theory of maximum a posteriori (MAP) estimation. Specifically, as unknown parameters, Θ should be treated as random variables in its posterior distribution P(Θ|hl(j),Γl) which is proportional to the product of Phl(j),ΓlΘ and PΘ. Next, to obtain the expected log-posterior QΘ,Θj, we should transform the log-joint distribution Phl(j),ΓlΘ to its expectation over the posterior distribution of Γl|hl(j),Θj.

The joint distribution Phlj,ΓlΘ of complete data can be obtained following (10) by taking hl as a known value hlj. Next, the equations of the posterior expectation Γl and covariance CΓl  of latent variable Γl can be calculated following (11) by taking w as a known value w(j). According to the estimated posterior expectation and covariance of latent variable Γl, the expected log-posterior of complete data over the posterior distribution of Γl|hl(j),Θj at iteration j is

QΘ,Θj=EΓl|hl(j),ΘjlnPhl(j),ΓlΘ+lnPΘ,    (13a)

where EΓl|hl(j),ΘjlnPhl(j),ΓlΘ is the expectation of the log-joint distribution over the posterior distribution of Γl|hl(j),Θj

EΓl|hl(j),ΘjlnPhl(j),ΓlΘ=-12lnl+dln2β-lnllnσ-L2lnCΓ-12lTr1σ2ΦlTΦl+CΓ-1CΓl+ΓlT1σ2ΦlTΦl+CΓ-1Γl-2ΓlT1σ2ΦlTxlw(j)+CΓ-1μΓ+1σ2xlw(j)Txlw(j)+μΓTCΓ-1μΓ

(13b)

and lnPΘ is log-hyperprior distribution (i.e., (9)) for Θ

lnPΘ=const-12τ+d+2lnCΓ+TrCΓ0CΓ-1+βμΓ-μΓ0TCΓ-1μΓ-μΓ0,     (13c)

where

const=βd/2⋅CΓ0d/2(2π)d/2⋅2τd/2⋅Γd(τ/2)

in which Γd(⋅) denotes the multivariate gamma function, d is the dimension of the distribution and equal to 3 in our quadratic degradation model, and const has no relationship with Θ that can be ignored in calculation. In Appendix C, we provide more details about the calculation of QΘ,Θj in E step.

The M step: Θ can be updated through arg maxΘ QΘ,Θj and obtain Θj+1=μΓj+1,CΓj+1,σ2j+1. The update equations are derived as follows

∂QΘ,Θj∂μΓ=0     ⇒

μΓj+1=1β+LβμΓ0+lΓl ,

∂QΘ,Θj∂CΓ=0     ⇒

CΓj+1=1L+d+τ+2βμΓj+1-μΓ0μΓj+1-μΓ0T+            CΓ0+lCΓl+lΓl-μΓΓl-μΓT,

∂QΘ,Θj∂σ2=0    ⇒

σ2j+1=1lnllxlwj-ΦlΓl2+TrΦlCΓlΦlT. (14)

We repeat the above procedures to update the model parameters until convergence is achieved. The convergence condition is RMSEj+1%<δ, where RMSEj+1 is the root-mean-square error calculated by comparing Θj+1 and Θj, and δ is the prespecified convergence threshold. It should be noted that for hyperparameters in our proposed algorithm, we set the parameters in hyperprior NIW distribution β=1 and τ=d+1 following common settings. μΓ0 and  CΓ0 are supposed to be consistent with the distribution of training data, and thus we take a batch of data for pre-training to detect their proper settings.

Property of the QOIEM algorithm

In this subsection, we focus on studying the convergence property of the proposed QOIEM algorithm in theory. The ultimate purpose of investigating the convergence of QOIEM algorithm is to prove that the parameters Θ=μΓ,CΓ,σ2 can gradually approach to their true values after iterations. It is equivalent to prove that the log-likelihood function lnLhlj,Θ are able to be maximized in each  iterationj i.e.,    lnLhlj+1,Θj+1≥ lnLhlj,Θj. We first prove lnLhlj,Θj+1≥ lnLhlj,Θj, and then prove lnLhlj+1,Θj+1≥lnLhlj,Θj+1.

- Firstly, in MAP-EM procedure, Θ is considered as a variable and according to the Bayes rule, lnLhlj,Θ can be transformed as

- lnLhlj,Θ=lnPhljΘPΘ=lnPhlj,Γl|ΘPΘ-lnPΓlhlj,Θ.

- (15)

- Then, we calculate the equation over the expectation of distribution Γl|hlj,Θj like the E step. Note that Lhlj,Θ and PΘ  has no relationship with Γl and thus we can obtain the following equation:

- lnLhlj,Θ=QΘ,Θj-RΘ,Θj,         (16)

- where RΘ,Θj represents EΓl|hl(j),ΘjlnPΓlhlj,Θ. Recall that the proof of convergence is equivalent to prove lnLhlj,Θj+1≥ lnLhlj,Θj , and QΘ,Θj will be maximized in the M step and thus QΘj+1Θ,Θj≥QΘj,Θj. Therefore, our goal is equivalent to prove the following inequality holds for arbitrary  Θ :

- RΘ,Θj-RΘj,Θj≤0.                 (17)

- Based on the expression of expectation, the left side of (17) can be transformed as

- ΓlPΓlhlj,ΘjlnPΓlhlj,ΘPΓlhlj,Θj=EΓl|hl(j),ΘjlnPΓlhlj,ΘPΓlhlj,Θj,

- (18)

- where PΓlhlj,Θ is the PDF and thus ΓlPΓlhlj,Θ=1 . Then, according to Jensen inequality, the proof of (17) can be obtained as follows:

- EΓl|hlj,ΘjlnPΓlhlj,ΘPΓlhlj,Θj≤lnEΓl|hlj,ΘjPΓlhlj,ΘPΓlhlj,Θj=lnΓlPΓlhlj,ΘjPΓlhlj,ΘPΓlhlj,Θj=lnΓlPΓlhlj,Θ=0,

- (19)

- where the equal sign is established if and only if Θ=Θj. After MAP-EM procedure, the variable Θ is updated as Θj+1 and therefore lnLhlj,Θj+1≥ lnLhlj,Θj.

- Secondly, according to the restrictions of estimation procedure for weight coefficient w in section C, the fitting error of HI (i.e., xlw-ΦlΓlj+1) has been minimized in estimation. Based on (10) with respect to  xlw-ΦlΓl, the log-likelihood of joint distribution Phlj+1,Γlj+1Θ of complete data increases correspondingly after updating w and therefore lnLhlj+1,Θj+1≥ lnLhlj,Θj+1.

- Finally, based on the discussion above, the convergence of the QOIEM algorithm is proved.

RUL prediction

After model parameters are estimated, we predict the RUL for in-service units. Compared with the historical units that contain the entire degradation processes until failure used in parameter estimation, sensor signals of the in-service units are only available until certain time point before failure. For an in-service unit q, we have the incomplete sensor signal matrix xq=xqt1,xqt2,⋯,xqtnqT,where nq is the number of available observations of unit q . For RUL prediction, we first calculate the posterior distribution Γq|hq , where hq is the HI obtained by hq=xqw, as follows

Γq|hq ∼N(μq, Σq),

where

Σq=ΦqTΦqσ2+CΓ-1-1,

μq=Σq1σ2ΦqTxqw+CΓ-1μΓ,                    (20)

Φq=ϕt1,ϕt2,⋯,ϕtnqT and tnq denotes the last observed time of the in-service unit q (see details in Appendix D).

Then, given the degradation status of the in-service unit q at a specific time tnq+t, i.e.,  gqtnq+t=ϕ(tnq+t)Γq, where ϕ(tnq+t)=1,tnq+t,tnq+t2, we obtain the distribution of the degradation status at time tnq+t as gqtnq+t∼Nμtnq+t, σtnq+t2, where μtnq+t=ϕ(tnq+t)μq and σtnq+t2=ϕ(tnq+t)ΣqϕT(tnq+t). In addition, we consider the variation of the failure threshold D. The distribution of D is written as D∼ND,σD2, where the mean value and variance can be estimated by the sample mean D and sample variance σD2 based on the HIs at the failure time of the historical units. According to the distribution of gqtnq+t and D, the probability that the degradation status exceeds the failure threshold at time t after tnq can be written as

FTq|hq=PTq≤thq=Pgqtnq+t≥D|hq=CDF-D+μnq+tσD2+σnq+t2=CDFξt,             (21)

where Tq is the RUL of the in-service unit q and CDF(∙) is the cumulative distribution function (CDF) of the standard normal distribution. Given   Tq≥0 , the conditional probability can be written as

P(Tq≤t|Tq≥0)=P(0≤Tq≤t|hq)P(Tq≥0|hq)=CDFξt-CDFξ01-CDFξ0.  (22)

Finally, the RUL of the in-service unit q is predicted by solving PTq≤tTq≥0=0.5 with variable t.

Numerical Experiments

In this section, we evaluate the effectiveness of our proposed method through numerical experiments. We conduct our proposed method on generated simulation dataset with degradation processes of multiple sensors, and then compare the RUL prediction results using our proposed method and the benchmark method under different levels of missing data for thorough evaluation.

Data generation

We randomly generate 1000 units with a linear degradation path, i.e., the underlying degradation process for each unit is

glt=Γl, 0+Γl, 1t,                           (23)

where the random effect parameter Γl=Γl, 0, Γl, 1T is generated from a bivariate normal distribution

Γl~NμΓ, CΓ.                                (24)

We describe the hyperprior distribution of μΓ and CΓ by a normal-inverse Wishart distribution

μΓ, CΓ~NμΓ|-12, CΓIWCΓ|10110.5,3.    (25)

Note that the probability of Γl, 1≤0 is less than 10-4, which is thus negligible. If any Γl, 1 with Γl, 1≤0 is generated, we reject the sample and generate a new one to ensure that the underlying degradation process of any unit is increasing. The failure threshold is set to be D=400 and we record the true failure time as τl according to τl=argmintglt≥D.

Fig. 4. The convergence process of the QOIEM algorithm by taking the parameter Γl=Γl, 0, Γl, 1T as a demonstration. (Note: Γl is a two-dimensional vector whose true values for all of the historical units are displayed on a plane in the form of red scattered points. The yellow ellipse represents the confidence ellipse of the multivariate normal distribution NμΓ, CΓ and its mean μΓ and covariance CΓ are being updated by the QOIEM algorithm. The plots from left to right use the estimated μΓ and CΓ at the initial iteration, an intermediate iteration, and the final iteration to draw the ellipse respectively.)

We generate the true HI by adding a random noise εlt to the degradation process hlt=glt+εlt, where εlt follows a normal distribution  εlt~N0,52. Each unit has four sensors with the true value of fusion coefficients w=w1,w2,w3,w4T=0.45,0.15,-0.4,0T. Four sensor signals are randomly generated by

xl,1t=Θl,11t-Θl,12sin0.05t+εl,1t,

xl,2t=Θl,21t+Θl,22sin0.1t+εl, 2t,

xl,3t=hlt-ω1xl,1t-ω2xl,2t/ω3,

xl,4t=Θl,41t+Θl,42+εl, 4t,                    (26)

where Θl,11,Θl,12,Θl,22,Θl,42~Uniform0,30, Θl,21,Θl,41~Uniform0,2 and  εl,1,εl,2,εl,4~N0,52. Here, the sensor signal xl,3t is generated from the true HI ylt subtracting the sensor signals xl,1t and xl,2t to satisfy hlt=xltw. Since the fusion parameter w4=0, the sensor signal xl,4t is not related to the underlying degradation process. Fig. 2 presents an example of the sensor signals and true HIs for three randomly generated units.

Fig. 2.  An example of the sensor signals and true HIs for three randomly generated units.

Model performance of ideal scenario

We randomly select part of generated units as the historical units, and the remaining units as the in-service units. Our aim is to use the historical units to fit our proposed model and predict the RUL of the in-service units for model evaluation.

To explore the required number of historical units for stable and accurate parameter estimation in our proposed model, we estimate the weight coefficient w using the QOIEM algorithm based on different number of historical units. Specifically, we randomly select L historical units to estimate w and repeat the above procedure 50 times to obtain the mean and standard deviation of our estimations. The results are shown in Fig. 3, where X-axis indicates the number of units involved in parameter estimation. Black dashed horizontal lines represent the true values of w of each sensor. Red solid lines represent the mean values of the estimated w. Blue dotted lines represent corresponding 95% confidence intervals of the estimated w. As the number of units involved in parameter estimation increases, the estimated w1,w2,w3 and w4 (red solid lines) are getting closer to their true values (black dashed horizontal lines) and their variances (blue dotted lines) keep decreasing. Therefore, to obtain the stable and accurate estimation of weight coefficient w, it is recommended that the number of historical units exceeds 40.

Fig. 3. The estimated fusion coefficients by considering different numbers of units. (Note: X-axis indicates the number of units involved in estimation. Black dashed horizontal lines represent the true values of coefficients of each sensor. Red solid lines represent the mean values of the estimated fusion coefficients by considering different number of units. Blue dotted lines represent the corresponding 95% confidence intervals.)

In our experiments, we choose 500 units from 1000 generated units as the historical units, and the remaining 500 units as the in-service units. Based on the historical units, we use the QOIEM algorithm to estimate the parameters Θ=μΓ,CΓ,σ2 of the degradation model and the weight coefficient w in the linear fusion function.

We present the convergence process of the proposed QOIEM algorithm in Fig. 4 by taking the parameter Γl=Γl, 0, Γl, 1T as a demonstration. Recall that the true value of Γl is a two-dimensional vector generated by (19) and (20) for each historical unit l. We display the true values of Γl for all of the 500 historical units on a plane in the form of red scattered points. In the QOIEM, we assume Γl~NμΓ, CΓ and its mean μΓ and covariance CΓ are updated by the EM algorithm. The yellow ellipses show the confidence ellipses of the multivariate normal distribution NμΓ, CΓ using the estimated μΓ and CΓ at the initial iteration, an intermediate iteration, and the final iteration respectively. It can be seen that the confidence ellipse gradually approaches the true values of Γl as the iteration goes and the final confidence ellipse (yellow ellipse) is dovetail with the true values of Γl (red scatters), which indicates superior convergence performance of the proposed QOIEM algorithm and high accuracy of the estimated parameters. Table Ⅰ shows the true values and estimated values of the model parameters. The estimated values are very close to the true ones, which verifies the efficiency of the proposed QOIEM algorithm.

TABLE I

Comparison of model parameters in numerical experiments.

| Parameters | True value | Estimated value |
| --- | --- | --- |
| w | (0.45,0.15, -0.4,0)T | (0.4503,0.1488, -0.4009,0)T |
| μΓ | -1,2T | -1.23,1.97T |
| CΓ | 10110.5 | 9.770.930.930.42 |
| σ2 | 25 | 24.65 |

After estimating model parameters, we predict the RUL of

Fig. 6. Comparison of RUL prediction results by the proposed and benchmark methods under various ratios of missing data. (Note: X-axis represents the level of actual RUL, Y-axis represents the mean value of the RUL prediction errors, and the vertical sticks represent corresponding standard errors.)

the in-service units to evaluate our model performance. We define the prediction error ϵq of the in-service unit q as the absolute difference between the predicted RUL Tq and the true RUL Tq of the in-service unit q divided by its true failure time τq:

ϵq= Tq- Tqτq.                                 (27)

We calculate ϵq, with q=1, …, Q, for all in-service units, and then obtain the corresponding mean values and the standard errors under different levels of actual RUL, where the standard error is defined as the one standard deviation of the prediction errors divided by the square root of the number of units. To show the effectiveness of the proposed method, we compare it with a benchmark method in [1], which constructs a data fusion-based HI for degradation modeling but ignores dependency and heterogeneity among units by assuming the units are independent with each other. Fig. 5 shows the comparison of predicted RUL by the proposed and benchmark methods under different levels of actual RUL. X-axis represents the level of actual RUL, for which level “40” refers to the in-service units that have actual RUL less than or equal to 40 and level “+∞” refers to all in-service units. Y-axis represents the mean value of the RUL prediction errors, and the vertical sticks represent corresponding standard errors. Here, the standard error is the one standard deviation of the RUL prediction errors divided by the square root of the number of units. We can find from this figure that the proposed method (green bars) has consistently lower RUL prediction errors than the benchmark method (red bars) under all levels of actual RUL. The standard errors of the proposed and benchmark methods both keep small under all levels of actual RUL.

Fig. 5. Comparison of RUL prediction results by the proposed and benchmark methods under different levels of actual RUL. (Note: X-axis represents the level of actual RUL, Y-axis represents the mean value of the RUL prediction errors, and the vertical sticks represent the corresponding standard errors).

Model performance of the scenarios with missing values

To evaluate the effectiveness of the proposed method for RUL prediction of units with missing values in sensor signals, we generate several scenarios with different ratios of missing values. Specifically, we randomly select a certain number of data from sensor signals of the historical units and assume the selected data are missing. Fig. 6 presents the RUL prediction results by the proposed and benchmark methods for scenarios with 10%, 20%, 30% and 50% missing values, respectively. The proposed method (green lines) has consistently smaller RUL prediction errors than the benchmark method (red lines) for all scenarios with missing values, because the proposed method considers the dependency and heterogeneity between units and shares the common information among units for handling missing data. Besides, the accuracy of the proposed method is relatively high and its superiority is more significant than the benchmark method when the ratio of missing data is larger than 20%, which verifies the efficiency and robustness of the proposed method.

Case Study

Dataset description

Fig. 7. Simplified diagram of engine simulated by C-MAPSS [39].

In real case study, we choose the degradation-based dataset of aircraft turbofan engines to validate our proposed model and algorithms. The dataset is originated from the C-MAPSS [27], which has been adopted in many PHM related researches, such as [1] and [41].

To demonstrate the C-MAPSS data in more details, the schematic diagram of an aircraft turbofan engine is displayed in Fig. 7, and the detailed information about the 21 available degradation sensor signals of aircraft turbofan engines under the same operational condition and failure mode is shown in Table Ⅱ. The dataset includes 100 historical units (i.e., L=100) and 100 in-service units (i.e., Q=100). The data of historical units include the complete observations of degradation processes for each unit until failure and there are 20,631 observations (i.e., l=1Lnl=20,631) in total. Meanwhile, the data of in-service units only have the observations up to some time point before failure and there are 13,096 observations (i.e., q=1Qnq=13,096) in total. Each observation contains 21 sensor signals from different monitors in aircraft turbofan engines. We use the historical units for model construction and parameter estimation, and the in-service units for RUL prediction. We compare the predicted RUL to the provided true RUL for model validation. The preprocessing procedure are implemented as in [1]: Considering the degradation signals should exhibit a consistent increasing (or decreasing) trend, 11 candidate sensors are selected out of 21 sensors, including T24, T50, P30, Nf, Ps30, Phi, NRf, BPR, htBleed, W31, and W32. Next, all sensor signals are preprocessed by conducting z-score normalization and logarithm transformation.

TABLE Ⅱ

Detailed information of the 21 sensors in the case study.

| Symbol | Description | Units |
| --- | --- | --- |
| T2 | Total temperature at fan inlet | °R |
| T24 | Total temperature at LPC outlet | °R |
| T30 | Total temperature at HPC outlet | °R |
| T50 | Total temperature at LPT outlet | °R |
| P2 | Pressure at fan inlet | psia |
| P15 | Total pressure in bypass-duct | psia |
| P30 | Total pressure at HPC outlet | psia |
| Nf | Physical fan speed | rpm |
| Nc | Physical core speed | rpm |
| epr | Engine pressure ratio (P50/P2) | -- |
| Ps30 | Static pressure at HPC outlet | psia |
| phi | Ratio of fuel flow to Ps30 | pps/psi |
| NRf | Corrected fan speed | rpm |
| NRc | Corrected core speed | rpm |
| BPR | rpm Bypass Ratio | -- |
| farB | Burner fuel-air ratio | -- |
| htBleed | Bleed Enthalpy | -- |
| Nf-dmd | Demanded fan speed | rpm |
| PCNfR_dmd | Demanded corrected fan speed | rpm |
| W31 | HPT coolant bleed | lbm/s |
| W32 | LPT coolant bleed | lbm/s |

Results and comparison

TABLE Ⅲ

Estimated weight coefficient w in the fusion model for each sensor.

| Name | T24 | T50 | P30 | Nf | Ps30 | phi |
| --- | --- | --- | --- | --- | --- | --- |
| Value | 0.1012 | 0.1126 | -0.0831 | 0.0094 | 0.1407 | -0.1293 |
| Name | NRf | BPR | htBleed | W31 | W32 |  |
| Value | 0.0276 | 0.1176 | 0.0873 | -0.0944 | -0.0965 |  |

TABLE Ⅳ

Estimated Θ in the degradation model.

| Parameter | Value |
| --- | --- |
| μΓ | 5.66e-01,-2.18e-03,3.64e-05T |
| CΓ | 5.08e-02-1.52e-04-5.01e-07-1.52e-042.70e-06-1.34e-08-5.01e-071.34e-082.54e-10 |
| σ2 | 0.0090 |

- We first estimate model parameters using data of historical units. Table Ⅲ lists the estimated weight coefficient w in the fusion model for each sensor. The positive and negative of the weight reflects the increasing or decreasing trend of each sensor, and the sum of the absolute value of each sensor weight is 1. The estimated value of parameter Θ=μΓ,CΓ,σ2 in the degradation model is given in Table Ⅳ.

Fig. 8. Sensor signals and the constructed HI of a randomly selected historical unit. (Note: the blue dots are sensor signals and the constructed HI. The red solid lines are the fitted degradation statuses.)

Fig. 8 shows the sensor signals and the constructed HI of a randomly selected historical unit. In the figure, the data of each sensor xl,j(t) is fitted by the quadratic polynomial degradation model that proposed in [24]:

xl,j(t)=γl,j0+γl,j1t+γl,j2t2+εl,j(t)                (28)

where l and j represent the index of unit and sensor respectively, γl,j0,γl,j1,γl,j2 are random-effect parameters and εl,j(t) is the noise term. Γl,j=γl,j0,γl,j1,γl,j2T ∈R3×1 is the vector of model parameters and follows a multivariate Gaussian distribution Γl,j∼NμΓ,j,CΓ,j. Parameters of the degradation model are estimated through EM algorithm similar to (9)-(14) but without data fusion. Next, to characterize the degradation process of each single sensor signal of historical unit l, we calculate the posterior distribution Γl,j|xl,j following (15) and obtain the degradation curves of each single sensor signal shown in figure, where xl,j=xl,jt1,xl,jt2,⋯,xl,jtnlT∈Rnl×1 denotes the matrix of xl,jt. It can be seen that the HI has better monotonicity and smaller fluctuations than any single sensor signal, which indicates the proposed method for HI construction is reasonable and effective.

Next, we predict the RUL of the in-service units and compare RUL prediction errors of the proposed method and benchmark method. Fig. 9 shows the comparison of the RUL prediction errors using our proposed method (green bars) and the benchmark method (red bars) under different actual RUL levels. It can be seen that our proposed method has consistently smaller RUL prediction errors than the benchmark method under all RUL levels. It indicates that the proposed method can make better use of the common information among units and achieve more accurate parameter estimation results through the QOIEM algorithm.

Fig. 9. Comparison of RUL prediction results by the proposed and benchmark methods under different levels of actual RUL. Here X-axis represents the level of actual RUL, Y-axis represents the mean value of the RUL prediction errors, and the vertical sticks represent corresponding standard errors.

TABLE Ⅴ

RUL prediction errors based on HI (the proposed method) and each single sensor signal.

| Name | T24 | T50 | P30 | Nf | Ps30 | phi |
| --- | --- | --- | --- | --- | --- | --- |
| Error | 0.117 | 0.099 | 0.104 | 0.151 | 0.093 | 0.107 |
| Name | NRf | BPR | htBleed | W31 | W32 | HI |
| Error | 0.210 | 0.091 | 0.115 | 0.127 | 0.120 | 0.084 |

To evaluate the effectiveness of constructing the HI, we further compare the proposed method with the benchmarks that use each single sensor signal for degradation modeling and prognostics respectively (i.e., (23)). These benchmarks ignore data fusion without the construction of HI. Table Ⅴ shows the RUL prediction errors based on each single sensor signal and the constructed HI. It can be seen that the benchmarks that only use a single sensor signal induce relatively larger RUL prediction errors than the proposed method. Meanwhile, we find the sensor with smaller RUL prediction error has higher weight coefficient in the fusion model of our proposed method (see Table Ⅲ), which means our fusion model for HI construction and its result are credible.

Model performance for handling missing data

- In order to reflect various modes of missing data in real engineering cases, we design three different modes of signal losses to validate the performance of the proposed method as follows:

- (A) Drop data point: Randomly select a certain number of signal data in historical units and assume they are missing;

- (B) Drop time step: Randomly select a certain number of time steps in historical units, and assume all sensor signals at the selected time steps are missing. Note that considering engineering practice, the last observed time steps of the units are not dropped.

- (C) Compound drop method: Combine the above modes A and B.  First, we drop a certain ratio (30% in our experiments) of signal data as mode A, and then we drop time steps as mode B by setting various drop ratios.

Fig. 10. Number of iterations in the QOIEM algorithm under loss mode A, B and C with the ratios of missing data from 10% to 80%.

- Fig. 10 presents the number of iterations in the QOIEM algorithm for parameters estimation under loss mode A, B and C with the ratios of missing data from 10% to 80%. The number of iterations can reflect the convergence performance in our QOIEM algorithm, where we set RMSEj+1%<0.1% as the breakout condition. As is shown in the figure, the number of iterations keeps low when the missing rate is under 60% and raises as the missing rate increases but keeps no more than 12, which indicates under different levels of missing data, our proposed QOIEM algorithm can still have great convergence. Moreover, the iteration times of loss mode B (orange line) is relatively low, which is owing to our quadratic degradation model can well fit the degradation sensor signals and thus some missing time steps have little impact on the parameter estimation in our proposed method.

TABLE VI

RUL prediction errors based on HI (the proposed method) and each single sensor signal under loss mode a.

| Missing rate: 0.1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name | T24 | T50 | P30 | Nf | Ps30 | phi |
| Error | 0.122 | 0.100 | 0.107 | 0.151 | 0.095 | 0.108 |
| Name | NRf | BPR | htBleed | W31 | W32 | HI |
| Error | 0.203 | 0.096 | 0.151 | 0.133 | 0.126 | 0.092 |
| Missing rate: 0.2 |  |  |  |  |  |  |
| Name | T24 | T50 | P30 | Nf | Ps30 | phi |
| Error | 0.147 | 0.110 | 0.122 | 0.164 | 0.098 | 0.110 |
| Name | NRf | BPR | htBleed | W31 | W32 | HI |
| Error | 0.219 | 0.110 | 0.182 | 0.152 | 0.140 | 0.105 |
| Missing rate: 0.3 |  |  |  |  |  |  |
| Name | T24 | T50 | P30 | Nf | Ps30 | phi |
| Error | 0.170 | 0.117 | 0.142 | 0.176 | 0.113 | 0.115 |
| Name | NRf | BPR | htBleed | W31 | W32 | HI |
| Error | 0.217 | 0.130 | 0.202 | 0.171 | 0.149 | 0.111 |
| Missing rate: 0.5 |  |  |  |  |  |  |
| Name | T24 | T50 | P30 | Nf | Ps30 | phi |
| Error | 0.227 | 0.143 | 0.156 | 0.208 | 0.139 | 0.127 |
| Name | NRf | BPR | htBleed | W31 | W32 | HI |
| Error | 0.288 | 0.182 | 0.244 | 0.244 | 0.191 | 0.134 |

- Table VI shows RUL prediction errors based on HI (the proposed method) and each single sensor signal with missing data under loss mode A. It can be seen that by conducting data fusion to construct HI, the RUL prediction errors are able to be decreased compared with each single sensor signal because the constructed HI contains more information for degradation modeling. Meanwhile, as the missing rate increases, the accuracy of RUL prediction by HI is more prominent than other results predicted by single sensor signals, which indicates that our proposed method gains more robustness for accurate RUL prediction under different ratios of missing data. The results under loss modes B and C are similar to loss mode A, and thus omitted here.

Fig. 11. Comparison of RUL prediction results by the proposed and benchmark methods under loss modes A, B and C and with different levels of missing data from 10% to 80%.

Fig. 11 shows the comparison of RUL prediction results by the proposed and benchmark methods under loss modes A, B and C with different levels with missing data from 10% to 80%. It can be seen that under these situations, the proposed method (green lines) still has lower RUL prediction errors than the benchmark method (red lines) because the proposed method shares the common information among units to effectively handle the issue of missing data. Specifically, a) under loss mode A, the RUL prediction errors of the two methods both increase significantly with the increasing loss rate of missing data. b) Under loss mode B, the RUL prediction errors of the two methods have no significant change. The possible explanation is that the quadratic model fits the data well, and thus the accurate parameter estimation of the quadratic model can still be achieved with the absence of some time steps. C) Under loss mode C, because the proposed method captures the dependency and heterogeneity among units under the same operational condition and failure mode, the RUL prediction errors of the proposed method keep stable when the loss rate of missing data is increasing. On the contrary, the RUL prediction errors of the benchmark method increases with the increasing loss rate of missing data. In conclusion, the above analysis verifies the robustness of the proposed method when missing data exist.

Conclusion

In real production processes, prognosis of machines plays an important role in the maintenance of system stability. However, multiple sensor signals with missing data pose challenges to RUL prediction of machines. To address this issue, this paper proposes a data-level fusion based multi-task learning method for prognostics of machines. Specifically, we adopt the quadratic polynomial model to fit the degradation processes of machines based on HIs that are constructed by linear fusion model. Considering both the dependency and heterogeneity  among machines under the same operational condition and failure mode, we innovatively integrate the MTL into the polynomial degradation model, and develop a unique QOIEM algorithm for parameter estimation. Finally, given the constructed HI and degradation model parameters, the RUL can be predicted correspondingly.  We conduct the proposed method to numerical experiments and real case study on C-MAPSS data for model validation. As a result, the proposed method has advantages of superior robustness for accurate RUL prediction under different ratios of missing data compared with state-of-the-art benchmarks. In addition, our method has great convergence for parameter estimation.

For future work, we considered two related topics. First, the HI in the proposed method is constructed by a linear fusion model, future works will focus on non-linear fusion for systems with high complexity. Second, as our method only consider one type of failure mode and operational condition for RUL prediction, in future research we will consider predicting RUL under multiple failure modes and operational conditions.

- Appendix A

- Setting time-step weight matrix cl

- Recall that for the formula hl=ΦlΓl+ϵl, ϵl∈Rnl×1 represents the vector of residual term ϵl=ϵlt1,ϵlt2,⋯,ϵltnlTof unit l, where residual term ϵlt is the difference between the constructed HI hlt and the degradation model ΦltΓl at time t. The degradation statuses at time points that are closer to failure time has higher effect on RUL prediction. Therefore, we set the weight clt for residual term ϵlt, and clt should follows

- clti+1>clti≥0,  i=1,2⋯,nl,  l=1,2,⋯,L.   (A1)

- In addition, each historical unit shares the same importance so that clt should be normalized:

- i=1nlclti=1,l=1,2,⋯,L.                 (A2)

- We assume the clt follows an arithmetic series 2clti=clti-1+clti+1 and can be presented as:

- clti=clt1+i-12-2clt1nlnl-1nl,i=1,2⋯,nl.   (A3)

- Finally, we obtain the weight coefficient matrix cl=diagclt1,⋯,cltnl∈Rnl×nl.

- Appendix B

- The variance of HIs in the failure threshold

- We present the calculation of the variance in the failure threshold of the constructed His based on historical units. Recall that xτ=xτ1, …,xτl, …,xτLT∈RL×m and xτl denotes the vector of sensor signals of unit l at its failure time τl. Recall that xl=xlt1,xlt2,…,xltnlT∈Rnl×m represents sensor signal matrix that contains complete data of  degradation process until failure of the historical unit l, and thus its failure time τl is equal to its last observed time point nl. The estimated failure threshold of the constructed HI (called failure threshold below) can be considered as xτw∈RL×1. Then, the mean of the failure threshold can be expressed as (1Txτw)/L, where 1∈R1×L denotes the column vector with all ones. The mean of squared failure threshold can be expressed as xτwTxτw/L. Therefore, the unbiased estimation of the variance in the failure threshold is calculated as

- (xτwTxτw-L((1Txτw)/L)2)/(L-1).        (B1)

- (B1) can be transformed to (wTxτTxτw-(wTxτT11Txτw)/L)/(L-1)=xτwTGxτw, in which G=I-1T1LL-1∈RL×L and I ∈RL×L is the identity matrix.

- Appendix C

- The expected log- posterior QΘ,Θj

In the E step of the QOIEM algorithm, we calculate the expected log-posterior QΘ,Θj based on the joint distribution Phlj,ΓlΘ and hyperprior distribution (i.e., (9)) P(Θ).

Note that the joint distribution Phlj,ΓlΘ is proportional to PhljΓl,ΘPΓlΘ according to the Bayes rule. PhljΓl,Θ is the combination of multiple Gaussian distributions hljt=xltwj∼NΦltΓl,σ2, with t=t1,t2,⋯,tnl, thus the distribution of observed data is

PhljΓl,Θ=lt12πσ2exp-12σ2xltwj   -ΦltΓl2.         (C1)

PΓlΘ is the probability density function (PDF) of multivariate Gaussian distribution NdμΓ,CΓ, which is

l12πdCΓexp-12Γl-μΓTCΓ-1Γl-μΓ,    (C2)

where d is the dimension of the distribution, which is equal to 3 in our quadratic degradation model. Then, the joint distribution of complete data can be written as (10) and

Zl=12πσ2nl12πdCΓ.                      (C3)

- As is discussed in above text, maximum a posterior (MAP) requires the log-joint distribution lnPhl(j),ΓlΘ and log- prior distribution lnP(Θ)(i.e., (13c)). lnPhl(j),ΓlΘ can be transformed as

- -12ld+nlln2π-lnllnσ-L2lnCΓ-12JΓl, (C4a)

- where

- JΓl=ΓlT1σ2ΦlTΦl+CΓ-1Γl-2ΓlT1σ2ΦlTxlwj+CΓ-1μΓ+1σ2xlwjTxlwj+μΓTCΓ-1μΓ.

- (C4b)

- For the calculation of expected log-likelihood  EΓl|hl(j),ΘjlnPhl(j),ΓlΘ, according to the posterior distribution Γl|hlj,Θj with expectation Γl and covariance CΓl given in (11) and use w(j) to replace w as a known variable, we can obtain the expectation of expressions with Γl in (C4b):

- EΓlT1σ2ΦlTΦl+CΓ-1Γl=Tr1σ2ΦlTΦl+CΓ-1CΓl+ΓlT1σ2ΦlTΦl+CΓ-1Γl,

- EΓlT1σ2ΦlTxlwj+CΓ-1μΓ=ΓlT1σ2ΦlTxlw(j)+CΓ-1μΓ.

- (C5)

- and thus obtain EΓl|hl(j),ΘjlnPhl(j),ΓlΘ in (13b).

- Finally, the QΘ,Θj can be acquired by adding EΓl|hl(j),ΘjlnPhl(j),ΓlΘ (i.e., (13b)) and  lnP(Θ) (i.e., (13c)).

- Appendix D

- The posterior distribution  Γq|hq

Recall that Γq is the parameter of the degradation model and hq is the constructed HI of the in-service unit q. We have the prior distribution of Γq, i.e., Γq∼NμΓ,CΓ. Since hq=ΦqΓq+ϵq with ϵq∼N0,σ2 and the likelihood function of hq|Γq can be given based on the conditional distribution hq|Γq∼N(ΦqΓq,σ2), the posterior distribution of Γq|hqis obtained as

PΓqhq∝PhqΓqPΓq∝exp1σ2hq-ΦqΓqThq-ΦqΓq+Γq-μΓTCΓ-1Γq-μΓ∝expΓqTΦqTΦqσ2+CΓ-1Γq-21σ2ΦqTxqw+CΓ-1μΓΓq∝expΓq-μqTΣq-1Γq-μq

(D1)

where

Σq=ΦqTΦqσ2+CΓ-1-1,

μq=Σq1σ2ΦqThq+CΓ-1μΓ.

Therefore, the posterior distribution Γq|hq is a multivariate normal distribution N(μq,Σq) with mean vector μq and covariance matrix Σq.

References

[1]	K. Liu and S. Huang, “Integration of data fusion methodology and degradation modeling process to improve prognostics,” IEEE Trans. Autom. Sci. Eng., vol. 13, no. 1, pp. 344–354, Jan. 2016.

[2]	X. Zhang, L. Tang, and J. Decastro, “Robust fault diagnosis of aircraft engines: a nonlinear adaptive estimation-based approach,” IEEE Trans. Control Syst. Technol., vol. 21, no. 3, pp. 861–868, May 2013.

[3]	N. Gebraeel, M. Lawley, R. Liu, and V. Parmeshwaran, “Residual life predictions from vibration-based degradation signals: a neural network approach,” IEEE Trans. Ind. Electron., vol. 51, no. 3, pp. 694–700, Jun. 2004.

[4]	A. Rai and S. H. Upadhyay, “Bearing performance degradation assessment based on a combination of empirical mode decomposition and k-medoids clustering,” Mech. Syst. Signal Process., vol. 93, pp. 16–29, Sep. 2017.

[5]	Hoke, A. Brissette, D. Maksimović, A. Pratt, and K. Smith, “Electric vehicle charge optimization including effects of lithium-ion battery

degradation,” in Proc. IEEE Veh. Power Propulsion Conf., Sep. 2011,

pp. 1–8.

[6]	F. Yang, D. Wang, Y. Zhao, K.-L. Tsui, and S. J. Bae, “A study of the relationship between coulombic efficiency and capacity degradation of commercial lithium-ion batteries,” Energy, vol. 145, pp. 486–495, Feb. 2018.

[7]	P. Wang, B. D. Youn, and C. Hu, “A generic probabilistic framework for structural health prognostics and uncertainty management,” Mech. Syst. Signal Process., vol. 28, pp. 622–637, Apr. 2012.

[8]	M. S. Kan, A. C. C. Tan, and J. Mathew, “A review on prognostic techniques for non-stationary and non-linear rotating systems,” Mech. Syst. Signal Process., vol. 62–63, pp. 1–20, Oct. 2015.

[9]	Z.-S. Ye and N. Chen, “The inverse gaussian process as a degradation model,” Technometrics, vol. 56, no. 3, pp. 302–311, Jul. 2014.

[10]	L. Hao, K. Liu, N. Gebraeel, and J. Shi, “Controlling the residual life distribution of parallel unit systems through workload adjustment,” IEEE Trans. Autom. Sci. Eng., vol. 14, no. 2, pp. 1042–1052, Apr. 2017.

[11]	N. Gebraeel, “Sensory-updated residual life distributions for components with exponential degradation patterns,” IEEE Trans. Autom. Sci. Eng., vol. 3, no. 4, pp. 382–393, Oct. 2006.

[12]	N. Gebraeel, A. Elwany, and J. Pan, “Residual life predictions in the absence of prior degradation knowledge,” IEEE Trans. Reliab., vol. 58, no. 1, pp. 106–117, Mar. 2009.

[13]	X. Liu, J. Li, K. N. Al-Khalifa, A. S. Hamouda, D. W. Coit, and E. A. Elsayed, “Condition-based maintenance for continuously monitored degrading systems with multiple failure modes,” IIE Trans., vol. 45, no. 4, pp. 422–435, Apr. 2013.

[14]	C. Hu, B. D. Youn, P. Wang, and J. Taek Yoon, “Ensemble of data-driven prognostic algorithms for robust prediction of remaining useful life,” Reliab. Eng. Syst. Saf., vol. 103, pp. 120–135, Jul. 2012.

[15]	M. Hajiha, X. Liu, and Y. Hong, “Degradation under dynamic operating conditions: modeling, competing processes and applications,” J. Qual. Technol., vol. 53, no. 4, pp. 347–368, Aug. 2021.

[16]	X. Fang, N. Z. Gebraeel, and K. Paynabar, “Scalable prognostic models for large-scale condition monitoring applications,” IISE Trans., vol. 49, no. 7, pp. 698–710, Jul. 2017.

[17]	X. Li, J. Qian, and G. Wang, “Fault prognostic based on hybrid method of state judgment and regression,” Adv. Mech. Eng., vol. 5, p. 149562, Jan. 2013.

[18]	R. Moghaddass and M. J. Zuo, “An integrated framework for online diagnostic and prognostic health monitoring using a multistate deterioration process,” Reliab. Eng. Syst. Saf., vol. 124, pp. 92–104, Apr. 2014.

[19]	R. Gouriveau and N. Zerhouni, “Connexionist-systems-based long term prediction approaches for prognostics,” IEEE Trans. Reliab., vol. 61, no. 4, pp. 909–920, Dec. 2012.

[20]	E. G. Julie and S. T. Selvi, “Development of energy efficient clustering protocol in wireless sensor network using neuro-fuzzy approach,” Sci. World J., vol. 2016, pp. 1–8, 2016.

[21]	R. E. Silva et al., “Proton exchange membrane fuel cell degradation prediction based on adaptive neuro-fuzzy inference systems,” Int. J. Hydrog. Energy, vol. 39, no. 21, pp. 11128–11144, Jul. 2014.

[22]	J. Xu, Y. Wang, and L. Xu, “PHM-oriented integrated fusion prognostics for aircraft engines based on sensor data,” IEEE Sens. J., vol. 14, no. 4, pp. 1124–1132, Apr. 2014.

[23]	F. Wang, J. Du, Y. Zhao, T. Tang, and J. Shi, “A deep learning based data fusion method for degradation modeling and prognostics,” IEEE Trans. Reliab., vol. 70, no. 2, pp. 775–789, Jun. 2021.

[24]	K. Liu, N. Z. Gebraeel, and J. Shi, “A data-level fusion model for developing composite health indices for degradation modeling and prognostic analysis,” IEEE Trans. Autom. Sci. Eng., vol. 10, no. 3, pp. 652–664, Jul. 2013.

[25]	C. Song, K. Liu, and X. Zhang, “Integration of data-level fusion model and kernel methods for degradation modeling and prognostic analysis,” IEEE Trans. Reliab., vol. 67, no. 2, pp. 640–650, Jun. 2018.

[26]	Z. Li, J. Wu, and X. Yue, “A shape-constrained neural data fusion network for health index construction and residual life prediction,” IEEE Trans. Neural Netw. Learn. Syst., vol. 32, no. 11, pp. 5022–5033, Nov. 2021.

[27]	Y. Liu, User’s Guide for the Commercial Modular Aero-Propulsion System Simulation (C-MAPSS), 2nd ed. Cleveland, OH, USA: NASA Glenn Research Center, Mar. 2012.

[28]	X.-S. Si, “An adaptive prognostic approach via nonlinear degradation modeling: application to battery data,” IEEE Trans. Ind. Electron., vol. 62, no. 8, pp. 5082–5096, Aug. 2015.

[29]	S. S. Y. Ng, Y. Xing, and K. L. Tsui, “A naive bayes model for robust remaining useful life prediction of lithium-ion battery,” Appl. Energy, vol. 118, pp. 114–123, Apr. 2014.

[30]	B. Sinopoli, L. Schenato, M. Franceschetti, K. Poolla, M. I. Jordan, and S. S. Sastry, “Kalman filtering with intermittent observations,” IEEE Trans. Autom. Control, vol. 49, no. 9, pp. 1453–1464, Sep. 2004.

[31]	S. Mohamed and T. Marwala, “Neural network based techniques for estimating missing data in databases,” 16th Annu. Symp. Pattern Recognit. Assoc. South Afr., Jan. 2005.

[32]	O. Anava, E. Hazan, and A. Zeevi, “Online time series prediction with

missing data,” in Proc. ICML, 2015, pp. 2191–2199

[33]	R. J. Aliaga, “Real-time estimation of zero crossings of sampled signals for timing using cubic spline interpolation,” IEEE Trans. Nucl. Sci., vol. 64, no. 8, pp. 2414–2422, Aug. 2017.

[34]	R. Caruana, “Multitask learning,” Mach. Learn., vol. 28, no. 1, pp. 41–75, Jul. 1997.

[35]	K. Yu, V. Tresp, and A. Schwaighofer, “Learning Gaussian processes

from multiple tasks,” in Proc. 22nd Int. Conf. Mach. Learn., Bonn,

Germany, 2005, pp. 1–8.

[36]	H.-P. Wan and Y.-Q. Ni, “Bayesian multi-task learning methodology for reconstruction of structural health monitoring data,” Struct. Health Monit., vol. 18, no. 4, pp. 1282–1309, Jul. 2019.

[37]	M. Kim, C. Song, and K. Liu, “Individualized degradation modeling and prognostics in a heterogeneous group via incorporating intrinsic covariate information,” IEEE Trans. Autom. Sci. Eng., pp. 1–16, 2021.

[38]	T. Shireen, C. Shao, H. Wang, J. Li, X. Zhang, and M. Li, “Iterative multi-task learning for time-series modeling of solar panel pv outputs,” Appl. Energy, vol. 212, pp. 654–662, Feb. 2018.

[39]	T. S. Kim and S. Y. Sohn, “Multitask learning for health condition identification and remaining useful life prediction: deep convolutional neural network approach,” J. Intell. Manuf., vol. 32, no. 8, pp. 2169–2179, Dec. 2021.

[40]	H. J. Ferreau, C. Kirches, A. Potschka, H. G. Bock, and M. Diehl, “QpOASES: a parametric active-set algorithm for quadratic programming,” Math. Program. Comput., vol. 6, no. 4, pp. 327–363, Dec. 2014.

[41]	E. Ramasso and A. Saxena, “Performance benchmarking and analysis of prognostic methods for cmapss datasets.,” Int. J. Progn. Health Manag., vol. 5, no. 2, pp. 1–15, 2014.

This work was supported in part by the National Science Foundation of China under Grant 72101148; in part by the Shanghai Sailing Program under Grant 21YF1420100, and in part by the National Science Foundation of Shanghai under Grant 22ZR1433000.

Di Wang is with the department of Industrial Engineering and Management, School of Mechanical Engineering, Shanghai Jiao Tong University, Shanghai, China (email: d.wang@sjtu.edu.cn)

Yuhui Wang is with the department of Industrial Engineering and Management, School of Mechanical Engineering, Shanghai Jiao Tong University, Shanghai, China (email: yh.wang@sjtu.edu.cn)

Xiaochen Xian is with the department of Department of Industrial and Systems Engineering, University of Florida, Gainesville, FL 32611, USA (email: xxian@ufl.edu)